Landfill Gas Primer – Safety & Health
An Overview for Environmental Health Professionals

ATSDR – Agency for Toxic Substances and Disease Registry

### How are people exposed to landfill gas?

People may be exposed to landfill gases either at the landfill or in their communities. Landfill gases may migrate from the landfill either above or below ground. Gases can move through the landfill surface to the ambient air. Once in the air, the landfill gases can be carried to the community with the wind. Odors from day-to-day landfill activities are indicative of gases moving above ground. Gases may also move through the soil underground and enter homes or utility corridors on or adjacent to the landfill. Figure 3-1 illustrates the movement of landfill gases and potential exposure pathways.

### Explosion Hazards

### Landfill gas may form an explosive mixture when it combines with air in certain proportions. This section provides information about:

- The conditions that must be met for landfill gas to pose an explosion hazard.

- The types of gases that may potentially pose explosion hazards.

- What can be done to assess if a landfill is posing an explosion hazard.

Figure 3-1: Potential Exposure Pathways to Landfill Gas

### When does landfill gas pose an explosion hazard?

The following conditions must be met for landfill gas to pose an explosion hazard:

Gas production. A landfill must be producing gas, and this gas must contain chemicals that are present at explosive levels.

Lower and Upper Explosive Limits (LEL and UEL) 
The concentration level at which gas has the potential to explode is called the explosive limit. The potential for a gas to explode is determined by its lower explosive limit (LEL) and upper explosive limit (UEL). The LEL and UEL are measures of the percent of a gas in the air by volume. At concentrations below its LEL and above its UEL, a gas is not explosive. However, an explosion hazard may exist if a gas is present in the air between the LEL and UEL and an ignition source is present.

- Gas migration. The gas must be able to migrate from the landfill. Underground pipes or natural subsurface geology may provide migration pathways for landfill gas.

- Gas collection in a confined space. The gas must collect in a confined space to a concentration at which it could potentially explode. A confined space might be a manhole, a subsurface space, a utility room in a home, or a basement. The concentration at which a gas has the potential to explode is defined in terms of its lower and upper explosive limits (LEL and UEL), as defined at right.

| Landfill Gas Explosions |  |
| --- | --- |
| Although landfill gas explosions are by no means common occurrences, a number of incidents known or suspected to have been caused by landfill gas explosions have been documented. |  |
| 1999 | An 8-year-old girl was burned on her arms and legs when playing in an Atlanta playground. The area was reportedly used as an illegal dumping ground many years ago. (Atlanta Journal-Constitution 1999) |
| 1994 | While playing soccer in a park built over an old landfill in Charlotte, North Carolina, a woman was seriously burned by a methane explosion. (Charlotte Observer 1994) |
| 1987 | Off-site gas migration is suspected to have caused a house to explode in Pittsburgh, Pennsylvania.(EPA 1991) |
| 1984 | Landfill gas migrated to and destroyed one house near a landfill in Akron, Ohio.Ten houses were temporarily evacuated. (EPA 1991) |
| 1983 | An explosion destroyed a residence across the street from a landfill in Cincinnati, Ohio. Minor injuries were reported. (EPA 1991) |
| 1975 | In Sheridan, Colorado, landfill gas accumulated in a storm drain pipe that ran through a landfill. An explosion occurred when several children playing in the pipe lit a candle, resulting in serious injury to all the children. (USACE 1984) |
| 1969 | Methane gas migrated from an adjacent landfill into the basement of an armory in Winston-Salem, North Carolina. A lit cigarette caused the gas to explode, killing three men and seriously injuring five others. (USACE 1984) |

### What types of gases can pose an explosion hazard?

Table 3-1 summarizes the potential explosion hazards posed by the important constituents of landfill gas. Keep in mind that methane is the most likely landfill gas constituent to pose an explosion hazard. Other flammable landfill gas constituents are unlikely to be present at concentrations high enough to pose an explosion hazard. However, the flammable NMOCs do contribute to total explosive hazard when combined with methane in a confined space.

| Table 3-1: Potential Explosion Hazards from Common Landfill Gas Components |  |
| --- | --- |
| Component | Potential to Pose an Explosion Hazard |
| Methane | Methane is highly explosive when mixed with air at a volume between its LEL of 5% and its UEL of 15%. At concentrations below 5% and above 15%, methane is not explosive. At some landfills, methane can be produced at sufficient quantities to collect in the landfill or nearby structures at explosive levels. |
| Carbon dioxide | Carbon dioxide is not flammable or explosive. |
| Nitrogen dioxide | Nitrogen dioxide is not flammable or explosive. |
| Oxygen | Oxygen is not flammable, but is necessary to support explosions. |
| Ammonia | Ammonia is flammable. Its LEL is 15% and its UEL is 28%. However, ammonia is unlikely to collect at a concentration high enough to pose an explosion hazard. |
| NMOCs | Potential explosion hazards vary by chemical. For example, the LEL of benzene is 1.2% and its UEL is 7.8%. However, benzene and other NMOCs alone are unlikely to collect at concentrations high enough to pose explosion hazards. |
| Hydrogen sulfide | Hydrogen sulfide is flammable. Its LEL is 4% and its UEL is 44%. However, in most landfills, hydrogen sulfide is unlikely to collect at a concentration high enough to pose an explosion hazard. |

### How can I assess whether a landfill in my community poses an explosion hazard?

The following checklist can help determine if a landfill may pose an explosion hazard. If your evaluation identifies the potential for an explosion, several actions can be taken to prevent harm to the community.

| Landfill Gas Explosion Hazard Checklist |  |
| --- | --- |
|  | Is the landfill producing gas and if so, how much? |
|  | Because methane and carbon dioxide are the main components of landfill gas and these chemicals are both odorless and colorless, monitoring data are necessary to answer this question |
|  | Is a landfill gas collection system in place? |
|  | Landfill gas collection systems reduce levels of gas migrating from the landfill to surrounding areas. |
|  | Is gas migrating from the landfill? |
|  | Off-site monitoring data may be necessary to answer this question. |
|  | If gas is migrating from the landfill and reaching structures, are there places for gas to collect? |
|  | Uncontrolled gases escaping from a landfill may migrate to structures on the landfill itself or in the surrounding area. However, the further a structure is from the landfill, the less likely it is that gases are migrating to it at concentrations great enough to pose an explosion threat. The most common places for gases to collect are basements, crawl spaces, or buried utility entry ports. Homes with basements, especially those with pipes or cracks in the basement that would allow gas to enter, are more likely to collect gases. |
|  | Is gas collecting at concentrations that are high enough to pose an explosion hazard? |
|  | Monitoring data are needed to answer this question. Caution should be used in selecting sampling equipment to ensure that an ignition source is not introduced into the area. |
|  | Is there an ignition source? |
|  | Gases can be ignited by many different sources, such as a furnace in the basement or a pilot light on a gas stove. Other sources may include candles, matches, cigarettes, or a spark. Because there are so many ignition sources, it is safest to assume that the potential for an ignition source is always present. |

### Asphyxiation Hazards

Landfill gas poses an asphyxiation hazard only if it collects in an enclosed space (e.g., a basement or utility corridor) at concentrations high enough to displace existing air and create an oxygen-deficient environment.

Any of the gases that comprise landfill gas can, either individually or in combination, create an asphyxiation hazard if they are present at levels sufficient to create an oxygen-deficient environment.

Carbon dioxide, which comprises 40% to 60% of landfill gas, may pose specific asphyxiation hazard concerns. Because it is denser than air, carbon dioxide that has escaped from a landfill and collected in a confined space, such as a basement or an underground utility corridor, may remain in the area for hours or days after the area has been opened to the air (e.g., after a manhole cover has been removed or a basement door opened). Carbon dioxide is colorless and odor-less and therefore not readily detectable. Carbon dioxide concentrations of 10% or more can cause unconsciousness or death. Lower concentrations may cause headache, sweating, rapid breathing, increased heartbeat, shortness of breath, dizziness, mental depression, visual disturbances, or shaking.

### Can the presence of odors trigger symptoms?

People in communities near landfills are often concerned about odors emitted from landfills. They say that these odors are a source of undesirable health effects or symptoms, such as headaches and nausea. At low-level concentrations—typically associated with landfill gas— it is unclear whether it is the constituent itself or its odors that trigger a response. Typically, these effects fade when the odor can no longer be detected. The box below describes the biology behind detecting odors.

Landfill gas odors are produced by bacterial or chemical processes and can emanate from both active or closed landfills. These odors can migrate to the surrounding community. Potential sources of landfill odors include sulfides, ammonia, and certain NMOCs, if present at concentrations that are high enough. Landfill odors may also be produced by the disposal of certain types of wastes, such as manures and fermented grains.

How do Humans Detect Odor? Humans can distinguish between 3,000 and 10,000 different odors. Although people commonly believe that they smell with their noses, the nose is only one part of the olfactory system that allows humans to distinguish smells. The nose serves as a vacuum that pulls in air and odorous chemicals, such as hydrogen sulfide. The air and odorous chemicals are warmed in the nasal cavity and then trapped in mucus surrounding the olfactory membrane. The olfactory membrane is an area smaller than 1 square inch located deep in the nasal cavity between the eyes. Odorous chemicals interact with receptors (chemoreceptors) found on small hairs (cilia) on the olfactory membrane. The receptors send messages about the odorous chemicals to the brain through the olfactory bulb. The brain then interprets and identifies the odor (Pacific Science Center 1999; Jacobs 1999). The sense of smell, just like the other senses of sight, hearing, taste, and touch, varies from person to person. One person may be able to smell an odor like hydrogen sulfide at extremely low concentrations, while another person in the same community or home cannot. Because of this variation, there is no true odor threshold value above which odors are unpleasant and below which odors are not noticeable.

Anatomy of Smell

- Sulfides. Hydrogen sulfide, dimethyl sulfide, and mercaptans are the three most common sulfides responsible for landfill odors. These gases produce a very strong rotten-egg smell—even at very low concentrations. Of these three sulfides, hydrogen sulfide is emitted from landfills at the highest rates and concentrations.

- Ammonia. Ammonia is another odorous landfill gas that is produced by the decomposition of organic matter in the landfill. Ammonia is common in the environment and an important compound for maintaining plant and animal life. People are exposed daily to low levels of ammonia in the environment from the natural breakdown of manure and dead plants and animals. Because ammonia is a commonly used as a household cleaner, most people are familiar with its smell.

- NMOCs. Some NMOCs, such as vinyl chloride and hydrocarbons, may also cause odors. In general, however, NMOCs are emitted at very low (trace) concentrations and are unlikely to pose a severe odor problem.

Table 3-3  lists some of the common landfill gas components and their odor thresholds.

| Table 3-3: Common Landfill Gas Components and Their Odor Thresholds |  |  |
| --- | --- | --- |
| Component | Odor Description | Odor Threshold (parts per billion) |
| Hydrogen Sulfide | Strong rotten egg smell | 0.5 to 1 |
| Ammonia | Pungent acidic or suffocating odor | 1,000 to 5,000 |
| Benzene | Paint-thinner-like odor | 840 |
| Dichloroethylene | Sweet, ether-like, slightly acrid odor | 85 |
| Dichloromethane | Sweet,chloroform-like odor | 205,000 to 307,000 |
| Ethylbenzene | Aromatic odor like benzene | 90 to 600 |
| Toluene | Aromatic odor like benzene | 10,000 to 15,000 |
| Trichloroethylene | Sweet, chloroform-like odor | 21,400 |
| Tetrachloroethylene | Sweet, ether-or chloroform-like odor | 50,000 |
| Vinyl Chloride | Faintly sweet odor | 10,000 to 20,000 |

Many people may find the odors emitted from a landfill offensive or unpleasant. In reaction to the odor, some people may experience nausea or headaches. Although such responses are undesirable, medical attention is usually not required. Often, symptoms such as headaches and nausea fade when the odor goes away. However, the effects on day-to-day life can be more lasting. Families living close to a landfill in Connecticut described frequent odor events as overwhelmingly disruptive. One family reported being awakened during predawn hours by a flood of nauseating air that persisted for 2 or more hours. The loss of sleep and the frustration from the frequent odor events greatly added to the level of stress in the family’s life. Although landfill odors may not associated with long-term adverse health effects or illness for most people, the added disruption and stress of day-to-day activities can greatly impact quality of life.

Source of information: http://www.atsdr.cdc.gov/HAC/landfill/html/ch3.html#1